Delphi Electronics & Safety

Powertrain Gas Product Line

J1939 Software Test Report

For

Chery JD MT22.3 ETC

V1.1

Date: 20 January 2016

Contents

1.	Soft ID	1

2.	ECU ID	1

3.	TLA	2

4.	Active Diagnostic Trouble Codes (DM1)	2

5.	Previously Active Diagnostic Trouble Codes (DM2)	2

6.	Diagnostic Data Clear/Reset Of Previously Active DTCs (DM3)	2

7.	Freeze Frame Parameters (DM4)	3

8.	Diagnostic Readiness 1 (DM5)	3

9.	Command Non-Continuously Monitored Test (DM7)、Test Results For Non-Continuously Monitored Systems (DM8)	3

10.	Diagnostic Data Clear/Reset For Active DTCs (DM11)	3

11.	Stop Start Broadcast (DM13)	4

# Soft ID

SoftID=”SW0001,TCA12345,01.01#AP0101,TCU33470,00.07#*BB0001,PH25111,10.14#OS0001,PH250062,05.02#*”

# ECU ID

ECU ID= “PH90215494,01.00*SN0400219*China*Engine Controller*delphi*”

# TLA

TLA = “ECU”

# Active Diagnostic Trouble Codes (DM1)

Set  J1939_DM1_Message[J1939_DM1_BUFFER_SIZE] =

{   0x00, 0xFF, 0x00, 0xFE, 0xCA, 0x00, 0x00, 0x97, 0x03, 0x8a, 0x00,

0x51, 0x63, 0x8a};

Number of current DTC =0

Set J1939_DM1_DTCs=0

Number of current DTC =1,2,3

Set J1939_DM1_DTCs=1

Set J1939_DM1_DTCs=2

Set J1939_DM1_DTCs=3

# Previously Active Diagnostic Trouble Codes (DM2)

Set J1939_DM2_Message[J1939_DM2_BUFFER_SIZE] =

{   0x00, 0xFF, 0x00, 0xFE, 0xCA, 0x00, 0x00, 0x97, 0x03, 0x8a, 0x00,

0x51, 0x63, 0x8a};

Number of previous DTC =0

Set NumOfPreActiveDTCs=0

Number of previous DTC =1,2,3

Set NumOfPreActiveDTCs=1,2,3

# Diagnostic Data Clear/Reset Of Previously Active DTCs (DM3)

  - Set J1939_DM3_Clr_PreActive_Dtcs =false

//means preactive DTCs was not cleared

  - Set J1939_DM3_Clr_PreActive_Dtcs =true

// means preactive DTCs was cleared

# Freeze Frame Parameters (DM4)

  - No fault

Set  DTC_Number_DM4 =0 // the number of DTC

  - Fault active

Set  DTC_Number_DM4 =1,2,3

# Diagnostic Readiness 1 (DM5)

Set the J1939_73_dm5={1,2,3,4,5,6,7,8}

Reports the diagnostics information that relates to diagnostic readiness.

# Command Non-Continuously Monitored Test (DM7)、Test Results For Non-Continuously Monitored Systems (DM8)

Request injector1 open:

- J1939_73_dm7 was set as {1,0,0,0,0,0,0,0}

- B_Inj0Req was set to 1 after the message

# Diagnostic Data Clear/Reset for Active DTCs (DM11)

  - J1939_DM11_Clr_Active_Dtcs =false // means active DTCs was not cleared

  - J1939_DM11_Clr_Active_Dtcs =true // means active DTCs was cleared

# Stop Start Broadcast (DM13)

  - Hold

  - Suspend

  - Temporary suspend